# Massive Mobility Charging Simulator — How It Works

Short guide to the Massive Mobility OCPP 1.6 Charge Point simulator: what it is, how parts connect, and how a session runs.

---

# 1. What it is

This is an **OCPP 1.6 Charge Point (EVSE) simulator**.

It acts like a real charger talking to a CMS (for example Massive Charging).

It is **not** a CMS / Central System.

The browser UI is only a local control panel for the simulated charger.

Brand in the simulator: **Massive Mobility / Massive-CP-Sim-16**

BootNotification identity defaults:

Field | Default

Vendor | `Massive Mobility`

Model | `Massive-CP-Sim-16`

Serial | `MASSIVE-CPS-{cpId}`

Firmware | `Massive-CPS-16.3.2.1`

Project folder:

`C:\Users\massi\Downloads\OCPP_Simulator\evse-ocpp16-console`

---

# 2. Main pieces (stack)

Part | Role | Tech

**Client** (`client/`) | Operator UI + 3D charge point | Vite, React, Three.js / React Three Fiber

**Server** (`server/`) | Simulator brain + OCPP client | Node.js, Express, Socket.IO, `ws`

**CMS** (external) | Real Central System | e.g. Massive `wss://…`

---

# 3. How to run

1. Open the project folder.

2. Run `.\start.ps1` (or `npm run install:all` then `npm run dev`).

3. Open the UI: **http://localhost:5173**

4. API runs on: **http://localhost:8787**

Vite proxies `/api` and `/socket.io` from the UI to the backend.

---

# 4. How everything is connected

┌──────────────────────────┐

│  Browser (UI :5173)      │

│  3D CP + operator panel  │

└────────────┬─────────────┘

│

│  REST  /api/...

│  Socket.IO  (live state, logs, OCPP trace)

▼

┌──────────────────────────┐

│  Simulator backend       │

│  (:8787)                 │

│  ChargePoint simulator   │

└────────────┬─────────────┘

│

│  OCPP 1.6 WebSocket

│  URL = {CSMS base}/{cpId}

│  (optional subprotocol ocpp1.6, Basic Auth)

▼

┌──────────────────────────┐

│  CMS / CSMS              │

│  e.g. Massive Charging   │

└──────────────────────────┘

Important:

The **browser never speaks OCPP**.

Only the **Node server** opens the OCPP WebSocket to the CMS.

UI commands go REST API → simulator translates them into OCPP messages.

---

# 5. What travels on each connection

## A. Browser ↔ Simulator (local)

Channel | Used for

**REST `/api/...`** | Create station, plug, start/stop, fault, power, auth mode, reconnect, reset

**Socket.IO** | Live charger state, meter values, OCPP message trace, logs

## B. Simulator ↔ CMS (real protocol)

Channel | Used for

**OCPP WebSocket** | BootNotification, Heartbeat, StatusNotification, Authorize, StartTransaction, MeterValues, StopTransaction, RemoteStart/Stop, Reset, Config, etc.

OCPP messages are JSON arrays:

`[2, messageId, Action, payload]` = CALL (request)

`[3, messageId, payload]` = CALLRESULT (success)

`[4, messageId, errorCode, description, details]` = CALLERROR

---

# 6. Commissioning a station

In the left **Stations** panel:

1. Enter **Charge Point ID** (must match the CMS device / path id).

2. Enter **CSMS WebSocket base** (without trailing slash), e.g. `wss://stageocpp.example.com/ocpp`.

3. Set connector count and per-outlet kW (and optional names).

4. Click **Connect EVSE**.

The simulator connects to:

`{baseUrl}/{cpId}`

Example:

`wss://stageocpp.example.com/ocpp/NDOFCACP0001`

---

# 7. What happens after connect

1. WebSocket opens to CMS.

2. Simulator sends **BootNotification** (vendor, model, serial, firmware).

3. CMS replies Accepted / Pending / Rejected + heartbeat interval.

4. Simulator sends **StatusNotification** for connector `0` (whole CP) and each gun `1…N`.

5. Simulator may announce connector info (name/type/power) via StatusNotification `info` and a DataTransfer.

6. **Heartbeat** starts on the CMS interval.

After that the station shows **online**.

---

# 8. How a charging session works

## Local start (from UI / 3D screen)

1. Select connector (C1, C2, …).

2. Plug cable (or auto-plug on start).

3. **Authorize** idTag (local list and/or CMS, depending on auth mode).

4. **StartTransaction** → CMS returns `transactionId`.

5. Status → **Charging**; **MeterValues** sent periodically.

6. **Stop** → StopTransaction → brief Finishing → **Available**.

## Remote start (from CMS / app)

1. CMS sends **RemoteStartTransaction**.

2. Simulator accepts and runs the same Authorize / StartTransaction path.

3. CMS can later send **RemoteStopTransaction**.

---

# 9. Multi-connector notes

Supports **1–4** outlets plus connector **0** (charge point).

Each outlet has its own status, meter, and transaction.

Many CMS platforms (including Massive) **block the same RFID** on two concurrent sessions (`Blocked` / `ConcurrentTx`).

For parallel charging, use a **different CMS-registered idTag** per connector.

The simulator will try alternate lab tags when the current one is already in use.

Default lab tags:

`CARD-7F2A91`

`FOB-ORBIT-44`

`TOKEN-MASSIVE-09`

Auth modes:

**Local or CMS** (default) — accept if either allows

**Local only** — local list only

**CMS only** — CMS Authorize must accept

---

# 10. UI layout — how controls connect

UI area | What it does | Reaches CMS?

**3D charge point** | Touch screen + physical soft keys, outlets, RFID | Yes, via API → OCPP

**Operator panel (2D)** | Easy start/stop/plug/E-stop, idTag, kW | Yes, same path

**Stations dock** | Commission / select / remove CP | Creates OCPP connection

**Bench controls** | Fault, suspend, auth mode, reconnect, reset | Mixed (local sim + OCPP)

**Message / log panes** | Show OCPP frames and logs | Display only

Flow for a button press:

`UI click → REST API → ChargePoint method → OCPP CALL → CMS → CALLRESULT → Socket.IO state update → UI refresh`

---

# 11. Important API endpoints (short)

Endpoint | Purpose

`GET /api/health` | Backend health

`POST /api/chargers` | Create / connect station

`DELETE /api/chargers/:cpId` | Remove station

`POST .../plug` | Plug / unplug cable

`POST .../start` | Local authorize + start

`POST .../stop` | Stop transaction

`POST .../emergency-stop` | E-stop

`POST .../fault` / `clear-fault` | Inject / clear fault

`POST .../power` | Set outlet kW

`POST .../auth-mode` | Switch auth mode

`POST .../reconnect` | Reopen OCPP socket

`POST .../reset` | Soft / Hard reset

---

# 12. What is local-only (not OCPP)

These stay inside the simulator / browser:

3D graphics and layout

Socket.IO live feed to the UI

Local RFID allow-list editing

Meter / SoC simulation knobs

Choosing which station is selected in the UI

The CMS only sees the **OCPP WebSocket messages**.

---

# 13. Quick troubleshooting

Symptom | Likely cause

UI up, station offline | Wrong base URL / cpId, CMS down, TLS/auth issue

`Authorize Invalid` | Tag not accepted by CMS; try Local or CMS mode, or a real RFID

`StartTransaction idTag Blocked` | Same RFID already charging on another outlet

Stuck on Finishing in CMS | Older builds; current build sends Available after stop — reconnect station

Boot / ChangeConfiguration show `connectorId: null` | Normal — those are charge-point level messages

Need per-outlet status | Look for StatusNotification with connectorId 1, 2, …

---

# 14. One-line summary

Browser controls the simulator; the simulator speaks OCPP to the CMS.

WebSocket for OCPP is only Backend ↔ CMS. REST + Socket.IO are only Browser ↔ Backend.

---

Massive Mobility Charging Simulator — OCPP 1.6 Charge Point — connection and operation overview.