# OCPP 1.6 — Practical Guide

A clearer, more detailed overview of OCPP 1.6 JSON (OCPP-J) for EV charge points and CMS backends. Written for use with the Massive Mobility Charging Simulator.

For exact field schemas and enums, always check the official Open Charge Alliance OCPP 1.6 specification. This guide focuses on how the protocol works in practice.

# 1. What is OCPP 1.6?

OCPP (Open Charge Point Protocol) lets an EV charger and a Central System exchange charging events in a standard way.

| Role | Also called | Responsibility |
| --- | --- | --- |
| Charge Point (CP) | EVSE, charger, station | Physical charger; connects out to CMS; reports status, starts/stops sessions, sends meter values |
| Central System (CSMS / CMS) | Backend, CPO platform | Accepts charger connections; authorizes tags; assigns transactionId; can remote-start/stop, configure, reset |

## Important facts

OCPP is a message protocol, not a programming language or framework.

In OCPP 1.6 JSON, transport is WebSocket.

The Charge Point is the WebSocket client. The CMS is the WebSocket server.

The charger usually connects to something like:

wss://cms.example.com/ocpp/{chargePointId}

There is also an older SOAP / HTTP variant of 1.6; modern CMS platforms (including Massive Charging stage) typically use JSON over WebSocket.

## Feature profiles (1.6)

OCPP 1.6 groups optional capability into profiles. Common ones:

| Profile | Typical use |
| --- | --- |
| Core | Boot, Heartbeat, Status, Authorize, Start/Stop, MeterValues, RemoteStart/Stop, Reset, Config, TriggerMessage, Unlock, ChangeAvailability |
| FirmwareManagement | UpdateFirmware, FirmwareStatusNotification |
| LocalAuthListManagement | SendLocalList, GetLocalListVersion, ClearCache |
| Reservation | ReserveNow, CancelReservation |
| SmartCharging | SetChargingProfile, ClearChargingProfile, GetCompositeSchedule |
| RemoteTrigger | TriggerMessage (often listed with Core in practice) |

A charger advertises support via configuration key SupportedFeatureProfiles.

# 2. Connection model

┌────────────────────┐         WebSocket (OCPP-J)         ┌────────────────────┐
│  Charge Point      │  ────────────────────────────────► │  CMS / CSMS        │
│  (client)          │     wss://…/ocpp/{cpId}            │  (server)          │
│                    │  ◄──────────────────────────────── │                    │
└────────────────────┘     CALL / CALLRESULT / CALLERROR  └────────────────────┘

## Handshake notes

Optional WebSocket subprotocol: ocpp1.6

Some CMS endpoints require the subprotocol; some reject it. The Massive Mobility simulator can toggle this.

Optional HTTP Basic Auth on the handshake (username/password).

After the socket opens, the CP must send BootNotification before normal operation.

## Who may send what

Either side can send a CALL (request). The other side answers with CALLRESULT or CALLERROR.

Examples:

CP → CMS: BootNotification, StartTransaction, MeterValues

CMS → CP: RemoteStartTransaction, ChangeConfiguration, Reset

# 3. Message format (OCPP-J)

Every frame is a JSON array.

| Type | Code | Shape | Meaning |
| --- | --- | --- | --- |
| CALL | 2 | [2, messageId, action, payload] | Request |
| CALLRESULT | 3 | [3, messageId, payload] | Success reply to that messageId |
| CALLERROR | 4 | [4, messageId, errorCode, errorDescription, errorDetails] | Failure reply |

## Example CALL

[2, "19223201", "Heartbeat", {}]

## Example CALLRESULT

[3, "19223201", { "currentTime": "2026-08-11T09:00:00.000Z" }]

## Example CALLERROR

[4, "19223201", "NotImplemented", "Action not supported", {}]

## Rules of thumb

messageId must be unique enough for the open connection (string).

action is the OCPP operation name (BootNotification, StartTransaction, …).

Payload is a JSON object; some conf responses are empty {}.

Common CALLERROR codes: NotImplemented, NotSupported, InternalError, ProtocolError, SecurityError, FormationViolation, PropertyConstraintViolation, OccurrenceConstraintViolation, TypeConstraintViolation, GenericError.

# 4. Boot / go-online sequence

Typical first minutes after connect:

1. WebSocket open to {baseUrl}/{chargePointId}

2. CP sends BootNotification

3. CMS returns status, currentTime, interval

4. If Accepted (or after Pending becomes Accepted), CP sends StatusNotification for:

- connector 0 (whole charge point)

- each connector 1…N

5. CP starts Heartbeat every interval seconds (CMS may also change this later via ChangeConfiguration)

## BootNotification.req (CP → CMS) — key fields

| Field | Meaning |
| --- | --- |
| chargePointVendor | Vendor name (e.g. Massive Mobility) |
| chargePointModel | Model (e.g. Massive-CP-Sim-16) |
| chargePointSerialNumber | Serial |
| firmwareVersion | Firmware string (e.g. Massive-CPS-16.3.2.1) |
| iccid / imsi | Optional modem identities |
| meterType / meterSerialNumber | Optional meter info |

## BootNotification.conf (CMS → CP)

| Field | Meaning |
| --- | --- |
| status | Accepted / Pending / Rejected |
| currentTime | CMS clock (ISO 8601) |
| interval | Heartbeat interval in seconds |

If Rejected, the CP should not continue normal operation on that connection.

# 5. Connectors and status

## connectorId

| Id | Meaning |
| --- | --- |
| 0 | Whole Charge Point |
| 1…N | Physical outlets / guns |

OCPP 1.6 does not have a standard “rename connector” message. Names are often carried in StatusNotification.info, vendor DataTransfer, or CMS inventory only.

## StatusNotification

Sent when connector (or CP) status changes, and also on boot / trigger.

Important fields:

| Field | Meaning |
| --- | --- |
| connectorId | 0 or 1…N |
| status | See table below |
| errorCode | NoError or fault code (GroundFailure, OverCurrentFailure, …) |
| info | Free text (≤50 chars in many stacks) |
| timestamp | When the status applied |
| vendorId / vendorErrorCode | Vendor extensions |

## Connector status values

| Status | Typical meaning |
| --- | --- |
| Available | Free, ready |
| Preparing | User/cable interaction before charging |
| Charging | Energy flowing (or session active and delivering) |
| SuspendedEV | EV asked to pause |
| SuspendedEVSE | Charger/CMS paused delivery |
| Finishing | Session ending; often waiting for cable removal |
| Reserved | Reserved for an idTag |
| Unavailable | Taken out of service |
| Faulted | Error / fault present |

## Typical happy-path status chain

Available → Preparing → Charging → Finishing → Available

Some CMS UIs stay on Finishing until they see Available again (and sometimes until unplug). Simulators often force Available shortly after Stop so the CMS does not stick.

# 6. Authorization and idTags

An idTag is a string identifying the driver credential: RFID card UID, app token, virtual card, etc.

## Authorize

CP → CMS: { "idTag": "CARD-7F2A91" }

CMS → CP: { "idTagInfo": { "status": "Accepted", "expiryDate": "...", "parentIdTag": "..." } }

## Common idTagInfo.status values

| Status | Meaning |
| --- | --- |
| Accepted | Allowed to charge |
| Blocked | Explicitly blocked |
| Expired | Tag expired |
| Invalid | Unknown / not valid |
| ConcurrentTx | Already has another concurrent transaction (many CMS platforms enforce this) |

## Where authorization can happen

1. Online Authorize to CMS

2. Authorization cache on the CP

3. Local authorization list (SendLocalList)

4. Simulator modes (Massive Mobility sim): Local only / CMS only / Local or CMS

Important: Even if a simulator accepts a lab tag locally, the CMS may still reject StartTransaction if that tag is not registered in the CMS, or if the same RFID is already charging on another connector (Blocked / ConcurrentTx).

# 7. Charging session flows

## A) Local start (RFID / HMI)

1. Cable plugged (optional depending on charger logic)

2. Authorize idTag (local and/or CMS)

3. StartTransaction → CMS returns transactionId

4. Status → Charging

5. Periodic MeterValues

6. StopTransaction (local stop, app stop, E-stop, EV disconnect, …)

7. Status → Finishing → Available

## B) Remote start (app / CMS)

1. CMS sends RemoteStartTransaction { idTag, connectorId? }

2. CP answers Accepted or Rejected

3. If accepted, CP runs Authorize (if configured) + StartTransaction

4. Later CMS may send RemoteStopTransaction { transactionId }

## StartTransaction.req — key fields

| Field | Meaning |
| --- | --- |
| connectorId | Outlet number (1…N) |
| idTag | Credential |
| meterStart | Wh (or meter unit) at start |
| timestamp | Start time |
| reservationId | Optional |

## StartTransaction.conf

| Field | Meaning |
| --- | --- |
| transactionId | Session id assigned by CMS (required for later Stop / MeterValues) |
| idTagInfo.status | May still be Invalid / Blocked even after Authorize |

## StopTransaction.req — key fields

| Field | Meaning |
| --- | --- |
| transactionId | From StartTransaction |
| idTag | Optional |
| meterStop | Wh at end |
| timestamp | Stop time |
| reason | Local, Remote, EVDisconnected, EmergencyStop, PowerLoss, … |
| transactionData | Optional sampled values |

## MeterValues

While charging, CP sends samples. Important ideas:

Controlled by config keys such as MeterValueSampleInterval (seconds) and MeterValuesSampledData

Common measurands:

Energy.Active.Import.Register, Power.Active.Import, Current.Import, Voltage, SoC

Each sample has timestamp + sampledValue[] with value, measurand, unit, context, location, phase

CMS billing / live power views usually depend on these messages.

# 8. Messages from Charge Point → CMS

| Message | Purpose | When |
| --- | --- | --- |
| BootNotification | Identify CP + firmware | After connect / after reset / after FW install |
| Heartbeat | Keep-alive + clock sync opportunity | Every interval seconds |
| StatusNotification | Connector / CP status | Boot, change, trigger |
| Authorize | Check idTag | Before start (and sometimes remote start) |
| StartTransaction | Open session | Local or remote start |
| MeterValues | Energy / power / SoC samples | During transaction (and sometimes clock-aligned) |
| StopTransaction | Close session | End of charge |
| DataTransfer | Vendor-specific payload | Custom integrations |
| DiagnosticsStatusNotification | Diagnostics upload progress | After GetDiagnostics |
| FirmwareStatusNotification | FW update progress | After UpdateFirmware |
| SecurityEventNotification | (security whitepaper / extensions) | Implementation-specific |

# 9. Messages from CMS → Charge Point

| Message | Purpose | CP typical response |
| --- | --- | --- |
| RemoteStartTransaction | Start from app | Accepted / Rejected |
| RemoteStopTransaction | Stop by transactionId | Accepted / Rejected |
| Reset | Soft or Hard reboot | Accepted / Rejected |
| UnlockConnector | Unlock cable | Unlocked / UnlockFailed / NotSupported |
| ChangeAvailability | Operative / Inoperative | Accepted / Rejected / Scheduled |
| GetConfiguration | Read keys | Key/value list + unknown keys |
| ChangeConfiguration | Write a key | Accepted / Rejected / RebootRequired / NotSupported |
| ClearCache | Clear auth cache | Accepted / Rejected |
| GetLocalListVersion | Local list version | { listVersion } |
| SendLocalList | Full / differential local auth list | Accepted / Failed / VersionMismatch |
| ReserveNow | Reserve connector for idTag | Accepted / Faulted / Occupied / Rejected / Unavailable |
| CancelReservation | Cancel reservation | Accepted / Rejected |
| TriggerMessage | Ask CP to send Boot/Status/MeterValues/… | Accepted / Rejected / NotImplemented |
| DataTransfer | Vendor command | Accepted / Rejected / UnknownVendorId / UnknownMessageId |
| GetDiagnostics | Upload diagnostics to location | { fileName? } then status notifications |
| UpdateFirmware | Download/install firmware from URL | empty conf; then FirmwareStatusNotification |
| SetChargingProfile | Smart charging limit schedule | Accepted / Rejected / NotSupported |
| ClearChargingProfile | Remove profile(s) | Accepted / Unknown |
| GetCompositeSchedule | Ask planned schedule | schedule object or rejected |

# 10. Configuration (Get / Change Configuration)

Configuration is a set of string key/value pairs on the CP.

## Examples of important Core keys

| Key | Meaning |
| --- | --- |
| HeartbeatInterval | Seconds between Heartbeats |
| MeterValueSampleInterval | Seconds between MeterValues while charging |
| MeterValuesSampledData | Comma-separated measurands |
| NumberOfConnectors | Usually read-only |
| AuthorizeRemoteTxRequests | Whether RemoteStart must Authorize first |
| LocalAuthorizeOffline | Allow local auth when offline |
| LocalPreAuthorize | Use local list / cache without waiting |
| ConnectionTimeOut | Seconds to wait for auth/start interaction |
| ResetRetries | Reset retry count |
| SupportedFeatureProfiles | Advertised profiles |
| WebSocketPingInterval | WS ping (implementation-dependent) |
| AuthorizationCacheEnabled | Use auth cache |
| LocalAuthListEnabled | Use local list |
| StopTransactionOnEVSideDisconnect | Auto-stop on unplug |
| UnlockConnectorOnEVSideDisconnect | Auto-unlock on unplug |

ChangeConfiguration.conf may return RebootRequired — meaning the value is stored but applies only after Reset.

# 11. Firmware update (FirmwareManagement)

1. CMS sends UpdateFirmware with:

- location (URL of firmware file)

- optional retrieveDate, retries, retryInterval

2. CP answers with empty conf {}

3. CP sends FirmwareStatusNotification with statuses such as:

- Downloading → Downloaded → Installing → Installed

- or DownloadFailed / InstallationFailed

- then often back to Idle

4. After install, CP typically reconnects and sends a new BootNotification with the updated firmwareVersion

Massive Mobility simulator firmware example: Massive-CPS-16.3.2.1

# 12. Minimal set for a basic working charger

## Must send (CP → CMS)

BootNotification

Heartbeat

StatusNotification

Authorize

StartTransaction

MeterValues

StopTransaction

## Must handle (CMS → CP)

RemoteStartTransaction

RemoteStopTransaction

Reset

GetConfiguration / ChangeConfiguration

TriggerMessage

ChangeAvailability

## Strongly useful next

UnlockConnector

ClearCache / Local list

DataTransfer

UpdateFirmware + FirmwareStatusNotification

ReserveNow / CancelReservation

Smart charging messages (if CMS uses them)

# 13. Quick glossary

| Term | Meaning |
| --- | --- |
| CP / Charge Point | The charger |
| CSMS / CMS | Central System backend |
| EVSE | Electric Vehicle Supply Equipment (often same idea as CP / connector context) |
| connectorId | 0 = whole CP; 1…N = outlets |
| idTag | RFID / app / virtual credential string |
| transactionId | Session id returned by CMS on StartTransaction |
| OCPP-J | OCPP over JSON + WebSocket |
| CALL / CALLRESULT / CALLERROR | Request / success / failure frames |
| Measurand | What a meter sample measures (energy, power, SoC, …) |
| Local list | Auth tags stored on the charger |
| Feature profile | Optional capability group in OCPP 1.6 |

# 14. Common pitfalls

| Symptom | Likely cause |
| --- | --- |
| Socket connects but CMS shows offline | BootNotification missing / Rejected; wrong cpId path |
| Authorize Invalid | Tag not in CMS; wrong auth mode |
| StartTransaction Blocked / ConcurrentTx | Same idTag already charging elsewhere |
| No live kW in CMS | MeterValues not sent, wrong measurands, or interval too high |
| Stuck on Finishing | CMS waiting for Available / unplug StatusNotification |
| connectorId: null on Boot / ChangeConfiguration in UI traces | Normal — those messages are charge-point level, not per-outlet |
| Subprotocol errors | CMS requires or forbids ocpp1.6 subprotocol |
| Config change ignored | Key returned RebootRequired and no Reset yet |

# 15. How this maps to Massive Mobility Charging Simulator

| Piece | Role |
| --- | --- |
| Browser UI | Operator panel only (REST + Socket.IO) |
| Simulator Node backend | Real OCPP WebSocket client to CMS |
| Massive Charging CMS | Real OCPP WebSocket server |

Default BootNotification identity in the simulator:

Vendor: Massive Mobility

Model: Massive-CP-Sim-16

Firmware: Massive-CPS-16.3.2.1

Remember: the browser never speaks OCPP. Only the simulator backend does.

OCPP 1.6 JSON practical guide for the Massive Mobility Charging Simulator. For normative field-level schema, refer to the official OCPP 1.6 specification from the Open Charge Alliance.