# Massive Mobility Simulator — Quick Manual

Site: https://massive-ocpp-simulator.onrender.com

*(First load after idle can take ~1 minute.)*

# Screen map

| Area | Use it for |
| --- | --- |
| Left — Chargers | Create / select a charge point |
| Center — 3D charger | Touch screen, RFID pad, guns |
| Center — Operator panel | Quick plug / start / stop buttons |
| Right — Bench | Auth, car battery, faults, power, reconnect |
| Bottom — Message Trace | Live OCPP + system logs |

Drag ⋮⋮, resize edges, or Dock panels. Hide bench frees width for the charger.

# Connect a charger (left panel)

Enter Charge Point ID (CMS ID).

Paste CSMS WebSocket base (no ID on the end) — site connects to {base}/{ID}.

Set kW and Connectors (1–4).

Leave ocpp1.6 subprotocol ON unless CMS rejects it.

Fill Basic Auth only if CMS needs WebSocket username/password (not RFID).

Click Connect EVSE → wait for online.

# Run a charge

Demo tags (Auth = Local or CMS): CARD-7F2A91, FOB-ORBIT-44, TOKEN-MASSIVE-09

Plug cable (HMI PLUG, Operator panel, or Bench).

Set idTag → START or tap the RFID pad.

Watch energy / cost on the screen.

STOP → UNPLUG.

For a real Massive RFID: Auth = CMS only, paste the registered tag, then start.

# 3D charger screen

| Key | Action |
| --- | --- |
| HOME | Status + energy / power / cost |
| SESSION | Transaction, idTag, SoC |
| OUTLETS | Pick connector C1…C4 |
| INFO | Model, AC/DC, CPU / RAM / temp |
| PLUG / START / STOP / CLEAR | Cable, session, clear fault |

Drag to orbit, scroll to zoom.

# Operator panel (under charger)

Plug / Start / Stop / RFID / Emergency / Clear fault · set Auth + idTag · change outlet kW.

# Bench controls (right)

| Section | Use |
| --- | --- |
| Charger hardware | Live AC/DC, CPU, RAM, ROM, temp (simulated) |
| Auth mode | Local or CMS · Local only · CMS only |
| Cable / Session | Plug, start, stop reason, emergency |
| Car configuration | Pack energy + capacity; fill by full / kWh / money / time → Set car config before start |
| Suspend / Faults | Pause states; inject or clear OCPP faults |
| Hardware | Connector type, name, kW, ₹/kWh tariff |
| Link | Reconnect (± subprotocol), soft/hard reset |

# Message Trace (bottom)

| Tab / filter | Shows |
| --- | --- |
| OCPP | → CMS / ← CMS frames |
| System | Connect / error logs |
| All / Station / Cn | Filter by scope |

Resize height or collapse. Use this to confirm Boot, Authorize, Start/Stop, MeterValues.

# Quick fixes

| Problem | Try |
| --- | --- |
| Not connecting | Check URL + ID; Basic Auth; Reconnect ± subprotocol |
| Authorize Invalid | Wrong tag, or switch Auth mode |
| Old UI | Ctrl+Shift+R or private window |
| Same ID in two tabs | Use one only |