# OCPP 1.6 — Message Guide

Focus: OCPP 1.6 JSON (OCPP-J) — how it works and every standard message type.

For field-level schemas/enums, use the official Open Charge Alliance OCPP 1.6 specification.

# 1. Protocol basics

| Item | Fact |
| --- | --- |
| Purpose | Standard talk between an EV Charge Point (CP) and a Central System (CMS / CSMS) |
| Transport (JSON) | WebSocket — CP is client, CMS is server |
| URL shape | wss://cms.example.com/ocpp/{chargePointId} |
| Optional handshake | Subprotocol ocpp1.6, HTTP Basic Auth |
| First action after open | CP sends BootNotification |

Either side can send a request (CALL). The other replies with CALLRESULT or CALLERROR.

## Frame shapes

| Type | Code | Shape |
| --- | --- | --- |
| CALL | 2 | [2, messageId, action, payload] |
| CALLRESULT | 3 | [3, messageId, payload] |
| CALLERROR | 4 | [4, messageId, errorCode, errorDescription, errorDetails] |

Common CALLERROR codes: NotImplemented, NotSupported, InternalError, ProtocolError, SecurityError, FormationViolation, PropertyConstraintViolation, OccurrenceConstraintViolation, TypeConstraintViolation, GenericError.

## Feature profiles (groups of messages)

| Profile | Messages (main) |
| --- | --- |
| Core | Boot, Heartbeat, Status, Authorize, Start/Stop, MeterValues, RemoteStart/Stop, Reset, Unlock, ChangeAvailability, Get/ChangeConfiguration, ClearCache, DataTransfer, TriggerMessage |
| FirmwareManagement | UpdateFirmware, FirmwareStatusNotification |
| LocalAuthListManagement | GetLocalListVersion, SendLocalList, ClearCache |
| Reservation | ReserveNow, CancelReservation |
| SmartCharging | SetChargingProfile, ClearChargingProfile, GetCompositeSchedule |
| RemoteTrigger | TriggerMessage |

# 2. All Charge Point → CMS messages

| Message | Purpose | Typical when |
| --- | --- | --- |
| BootNotification | Identify CP (vendor, model, serial, firmware) | After connect / reset / FW install |
| Heartbeat | Keep-alive; CMS returns time | Every HeartbeatInterval seconds |
| StatusNotification | Report CP / connector status + errorCode | Boot, status change, trigger |
| Authorize | Ask if idTag may charge | Before start (and often remote start) |
| StartTransaction | Open a charging session | After auth / remote start |
| MeterValues | Energy, power, current, voltage, SoC samples | During (and sometimes outside) transaction |
| StopTransaction | Close session + final meter / reason | End of charge |
| DataTransfer | Vendor-specific payload (vendorId, messageId, data) | Custom extensions |
| DiagnosticsStatusNotification | Diagnostics upload progress | After GetDiagnostics |
| FirmwareStatusNotification | Firmware download/install progress | After UpdateFirmware |
| SecurityEventNotification | Security events (security whitepaper / stacks that support it) | Implementation-specific |

## Key payloads (CP → CMS)

BootNotification.req — chargePointVendor, chargePointModel, chargePointSerialNumber, firmwareVersion, optional modem/meter fields

BootNotification.conf — status (Accepted / Pending / Rejected), currentTime, interval (heartbeat seconds)

StatusNotification.req — connectorId (0 = whole CP, 1…N = outlet), status, errorCode, optional info, timestamp, vendorId, vendorErrorCode

Authorize.req — { idTag }

Authorize.conf — idTagInfo.status (Accepted, Blocked, Expired, Invalid, ConcurrentTx)

StartTransaction.req — connectorId, idTag, meterStart, timestamp, optional reservationId

StartTransaction.conf — transactionId (required for later stop/meters), idTagInfo

MeterValues.req — connectorId, optional transactionId, meterValue[] (timestamp + sampledValue[] with measurand/unit/context)

StopTransaction.req — transactionId, meterStop, timestamp, optional idTag, reason, transactionData

FirmwareStatusNotification.req — status (Idle, Downloading, Downloaded, Installing, Installed, DownloadFailed, InstallationFailed, …)

DiagnosticsStatusNotification.req — status (Idle, Uploaded, UploadFailed, Uploading)

# 3. All CMS → Charge Point messages

| Message | Purpose | Typical conf |
| --- | --- | --- |
| RemoteStartTransaction | Start from app/CMS | Accepted / Rejected |
| RemoteStopTransaction | Stop by transactionId | Accepted / Rejected |
| Reset | Soft or Hard reboot | Accepted / Rejected |
| UnlockConnector | Unlock cable on connector | Unlocked / UnlockFailed / NotSupported |
| ChangeAvailability | Operative / Inoperative (CP or connector) | Accepted / Rejected / Scheduled |
| GetConfiguration | Read config keys | key/value list + unknown key list |
| ChangeConfiguration | Write one config key | Accepted / Rejected / RebootRequired / NotSupported |
| ClearCache | Clear authorization cache | Accepted / Rejected |
| GetLocalListVersion | Local auth list version | { listVersion } |
| SendLocalList | Full or differential local auth list | Accepted / Failed / VersionMismatch |
| ReserveNow | Reserve connector for idTag | Accepted / Faulted / Occupied / Rejected / Unavailable |
| CancelReservation | Cancel reservation | Accepted / Rejected |
| TriggerMessage | Ask CP to send Boot / Status / MeterValues / Heartbeat / FW or Diagnostics status | Accepted / Rejected / NotImplemented |
| GetDiagnostics | Upload diagnostics to a location URI | optional fileName; then status notifications |
| UpdateFirmware | Download/install firmware from URL | empty {}; then FirmwareStatusNotification |
| SetChargingProfile | Apply smart-charging limit schedule | Accepted / Rejected / NotSupported |
| ClearChargingProfile | Remove profile(s) | Accepted / Unknown |
| GetCompositeSchedule | Ask planned power schedule | schedule object or rejected |
| DataTransfer | Vendor command to CP | Accepted / Rejected / UnknownVendorId / UnknownMessageId |

## Key payloads (CMS → CP)

RemoteStartTransaction.req — idTag, optional connectorId, optional chargingProfile

RemoteStopTransaction.req — transactionId

Reset.req — type: Soft / Hard

ChangeAvailability.req — connectorId, type: Operative / Inoperative

ChangeConfiguration.req — key, value (strings)

SendLocalList.req — listVersion, updateType (Full / Differential), localAuthorizationList[]

ReserveNow.req — connectorId, expiryDate, idTag, reservationId, optional parentIdTag

TriggerMessage.req — requestedMessage, optional connectorId

UpdateFirmware.req — location, optional retrieveDate, retries, retryInterval

GetDiagnostics.req — location, optional retries, retryInterval, startTime, stopTime

SetChargingProfile.req — connectorId, csChargingProfiles (purpose, stackLevel, schedule periods, …)

ClearChargingProfile.req — optional filters: id, connectorId, chargingProfilePurpose, stackLevel

GetCompositeSchedule.req — connectorId, duration, optional chargingRateUnit

# 4. Connector status values (StatusNotification)

| Status | Meaning |
| --- | --- |
| Available | Free |
| Preparing | Cable/user prep before charge |
| Charging | Session delivering (or active charge) |
| SuspendedEV | EV paused |
| SuspendedEVSE | Charger/CMS paused |
| Finishing | Ending; often wait for unplug |
| Reserved | Reserved |
| Unavailable | Out of service |
| Faulted | Error present |

connectorId 0 = whole charge point; 1…N = outlets.

Happy path: Available → Preparing → Charging → Finishing → Available

# 5. Session message flow

## Local start (RFID / HMI)

1. Authorize

2. StartTransaction → get transactionId

3. StatusNotification → Charging

4. MeterValues (repeat)

5. StopTransaction

6. StatusNotification → Finishing → Available

## Remote start

1. CMS RemoteStartTransaction

2. CP may Authorize

3. CP StartTransaction … same as above

4. CMS may RemoteStopTransaction

## Stop reasons (StopTransaction)

Local, Remote, EVDisconnected, EmergencyStop, PowerLoss, HardReset, SoftReset, Reboot, Unauthorized, DeAuthorized, Other, …

# 6. MeterValues (what gets sent)

Controlled by config such as MeterValueSampleInterval, MeterValuesSampledData.

Common measurands:

| Measurand | Typical meaning |
| --- | --- |
| Energy.Active.Import.Register | Energy Wh register |
| Power.Active.Import | Instant power |
| Current.Import | Current |
| Voltage | Voltage |
| SoC | EV state of charge % |

Each sample: timestamp + sampledValue[] (value, measurand, unit, context, location, phase).

# 7. Configuration messages

GetConfiguration / ChangeConfiguration read and write string keys on the CP.

Important Core keys:

| Key | Role |
| --- | --- |
| HeartbeatInterval | Heartbeat period (s) |
| MeterValueSampleInterval | Meter sample period (s) |
| MeterValuesSampledData | Measurands list |
| NumberOfConnectors | Outlet count |
| AuthorizeRemoteTxRequests | RemoteStart must Authorize? |
| LocalAuthorizeOffline / LocalPreAuthorize | Offline / local auth behavior |
| ConnectionTimeOut | Auth/start timeout |
| SupportedFeatureProfiles | Advertised profiles |
| AuthorizationCacheEnabled / LocalAuthListEnabled | Cache / local list |
| StopTransactionOnEVSideDisconnect | Auto-stop on unplug |
| UnlockConnectorOnEVSideDisconnect | Auto-unlock on unplug |

ChangeConfiguration.conf = Accepted | Rejected | RebootRequired | NotSupported

# 8. Firmware & diagnostics message flow

UpdateFirmware (CMS) → CP FirmwareStatusNotification

Downloading → Downloaded → Installing → Installed (or *Failed) → often Idle, then new BootNotification

GetDiagnostics (CMS) → CP DiagnosticsStatusNotification

Uploading → Uploaded (or UploadFailed) → Idle

# 9. Smart charging messages

| Message | Role |
| --- | --- |
| SetChargingProfile | Install TxProfile / TxDefaultProfile / ChargePointMaxProfile schedule |
| ClearChargingProfile | Remove matching profiles |
| GetCompositeSchedule | Return combined planned limit for a connector |

Limits from profiles typically cap power during MeterValues / charging.

# 10. Minimal message set for a working charger

CP must send: BootNotification, Heartbeat, StatusNotification, Authorize, StartTransaction, MeterValues, StopTransaction

CP must handle: RemoteStartTransaction, RemoteStopTransaction, Reset, GetConfiguration, ChangeConfiguration, TriggerMessage, ChangeAvailability

Next layer: UnlockConnector, ClearCache, SendLocalList, DataTransfer, UpdateFirmware + FirmwareStatusNotification, GetDiagnostics + DiagnosticsStatusNotification, ReserveNow / CancelReservation, Set/ClearChargingProfile, GetCompositeSchedule

# 11. Quick pitfalls

| Symptom | Related messages / cause |
| --- | --- |
| CMS stays offline | Missing/Rejected BootNotification; wrong cpId path |
| Can’t start | Authorize / StartTransaction Invalid / Blocked / ConcurrentTx |
| No live kW | Missing MeterValues or wrong measurands |
| Stuck Finishing | No StatusNotification Available after stop |
| Config ignored | ChangeConfiguration returned RebootRequired |
| Subprotocol fail | Handshake requires or forbids ocpp1.6 |