# Pier OCPP 1.6 Simulator — How It Works

Short guide to the Pier Charge Point lab: what it is, how parts connect, and how a session runs.

# 1. What Pier is

Pier is an OCPP 1.6 Charge Point (EVSE) simulator.

It acts like a real charger talking to a CMS (for example Massive Charging).

It is not a CMS / Central System.

The browser UI is only a local control panel for the simulated charger.

Brand in the lab: Quillgrid Systems / Pier-16H.

Project folder:

C:\Users\massi\Downloads\OCPP_Simulator\evse-ocpp16-console

# 2. Main pieces (stack)

| Part | Role | Tech |
| --- | --- | --- |
| Client (client/) | Operator UI + 3D charge point | Vite, React, Three.js / React Three Fiber |
| Server (server/) | Simulator brain + OCPP client | Node.js, Express, Socket.IO, ws |
| CMS (external) | Real Central System | e.g. Massive wss://… |

# 3. How to run

Open the project folder.

Run .\start.ps1 (or npm run install:all then npm run dev).

Open the UI: http://localhost:5173

API runs on: http://localhost:8787

Vite proxies /api and /socket.io from the UI to the backend.

# 4. How everything is connected

┌──────────────────────────┐

│  Browser (UI :5173)      │

│  3D CP + operator panel  │

└────────────┬─────────────┘

│

│  REST  /api/...

│  Socket.IO  (live state, logs, OCPP trace)

▼

┌──────────────────────────┐

│  Pier backend (:8787)    │

│  ChargePoint simulator   │

└────────────┬─────────────┘

│

│  OCPP 1.6 WebSocket

│  URL = {CSMS base}/{cpId}

│  (optional subprotocol ocpp1.6, Basic Auth)

▼

┌──────────────────────────┐

│  CMS / CSMS              │

│  e.g. Massive Charging   │

└──────────────────────────┘

Important:

The browser never speaks OCPP.

Only the Pier Node server opens the OCPP WebSocket to the CMS.

UI commands go Pier API → Pier translates them into OCPP messages.

# 5. What travels on each connection

## A. Browser ↔ Pier (local)

| Channel | Used for |
| --- | --- |
| REST `/api/...` | Create station, plug, start/stop, fault, power, auth mode, reconnect, reset |
| Socket.IO | Live charger state, meter values, OCPP message trace, logs |

## B. Pier ↔ CMS (real protocol)

| Channel | Used for |
| --- | --- |
| OCPP WebSocket | BootNotification, Heartbeat, StatusNotification, Authorize, StartTransaction, MeterValues, StopTransaction, RemoteStart/Stop, Reset, Config, etc. |

OCPP messages are JSON arrays:

[2, messageId, Action, payload] = CALL (request)

[3, messageId, payload] = CALLRESULT (success)

[4, messageId, errorCode, description, details] = CALLERROR

# 6. Commissioning a station

In the left Stations panel:

Enter Charge Point ID (must match the CMS device / path id).

Enter CSMS WebSocket base (without trailing slash), e.g. wss://stageocpp.example.com/ocpp.

Set connector count and per-outlet kW (and optional names).

Click Connect EVSE.

Pier connects to:

{baseUrl}/{cpId}

Example:

wss://stageocpp.example.com/ocpp/NDOFCACP0001

# 7. What happens after connect

WebSocket opens to CMS.

Pier sends BootNotification (vendor, model, serial, firmware).

CMS replies Accepted / Pending / Rejected + heartbeat interval.

Pier sends StatusNotification for connector 0 (whole CP) and each gun 1…N.

Pier may announce connector info (name/type/power) via StatusNotification info and a DataTransfer.

Heartbeat starts on the CMS interval.

After that the station shows online.

# 8. How a charging session works

## Local start (from Pier UI / 3D screen)

Select connector (C1, C2, …).

Plug cable (or Pier auto-plugs on start).

Authorize idTag (local list and/or CMS, depending on auth mode).

StartTransaction → CMS returns transactionId.

Status → Charging; MeterValues sent periodically.

Stop → StopTransaction → brief Finishing → Available.

## Remote start (from CMS / app)

CMS sends RemoteStartTransaction.

Pier accepts and runs the same Authorize / StartTransaction path.

CMS can later send RemoteStopTransaction.

# 9. Multi-connector notes

Pier supports 1–4 outlets plus connector 0 (charge point).

Each outlet has its own status, meter, and transaction.

Many CMS platforms (including Massive) block the same RFID on two concurrent sessions (Blocked / ConcurrentTx).

For parallel charging, use a different CMS-registered idTag per connector.

Pier will try alternate lab tags when the current one is already in use.

Default lab tags:

CARD-7F2A91

FOB-ORBIT-44

TOKEN-QUILL-09

Auth modes:

Local or CMS (default) — accept if either allows

Local only — local list only

CMS only — CMS Authorize must accept

# 10. UI layout — how controls connect

| UI area | What it does | Reaches CMS? |
| --- | --- | --- |
| 3D charge point | Touch screen + physical soft keys, outlets, RFID | Yes, via Pier API → OCPP |
| Operator panel (2D) | Easy start/stop/plug/E-stop, idTag, kW | Yes, same path |
| Stations dock | Commission / select / remove CP | Creates OCPP connection |
| Bench controls | Fault, suspend, auth mode, reconnect, reset | Mixed (local sim + OCPP) |
| Message / log panes | Show OCPP frames and logs | Display only |

Flow for a button press:

UI click → REST API → ChargePoint method → OCPP CALL → CMS → CALLRESULT → Socket.IO state update → UI refresh

# 11. Important API endpoints (short)

| Endpoint | Purpose |
| --- | --- |
| GET /api/health | Backend health |
| POST /api/chargers | Create / connect station |
| DELETE /api/chargers/:cpId | Remove station |
| POST .../plug | Plug / unplug cable |
| POST .../start | Local authorize + start |
| POST .../stop | Stop transaction |
| POST .../emergency-stop | E-stop |
| POST .../fault / clear-fault | Inject / clear fault |
| POST .../power | Set outlet kW |
| POST .../auth-mode | Switch auth mode |
| POST .../reconnect | Reopen OCPP socket |
| POST .../reset | Soft / Hard reset |

# 12. What is local-only (not OCPP)

These stay inside Pier / browser:

3D graphics and layout

Socket.IO live feed to the UI

Local RFID allow-list editing

Meter / SoC simulation knobs

Choosing which station is selected in the UI

The CMS only sees the OCPP WebSocket messages.

# 13. Quick troubleshooting

| Symptom | Likely cause |
| --- | --- |
| UI up, station offline | Wrong base URL / cpId, CMS down, TLS/auth issue |
| Authorize Invalid | Tag not accepted by CMS; try Local or CMS mode, or a real RFID |
| StartTransaction idTag Blocked | Same RFID already charging on another outlet |
| Stuck on Finishing in CMS | Older builds; current Pier sends Available after stop — reconnect station |
| Boot / ChangeConfiguration show connectorId: null | Normal — those are charge-point level messages |
| Need per-outlet status | Look for StatusNotification with connectorId 1, 2, … |

# 14. One-line summary

Browser controls Pier; Pier speaks OCPP to the CMS.

WebSocket for OCPP is only Backend ↔ CMS. REST + Socket.IO are only Browser ↔ Backend.

Pier OCPP 1.6 Charge Point lab — connection and operation overview.