# OCPP 1.6 — Short Guide

Practical cheat sheet for OCPP 1.6 JSON over WebSocket, for use with the Massive Mobility Charging Simulator. For full schemas, see the official Open Charge Alliance OCPP 1.6 spec.

# Roles

| Role | Does what |
| --- | --- |
| Charge Point (CP) | Charger. WebSocket client. Reports status, starts/stops, sends meters |
| CMS / CSMS | Backend. WebSocket server. Authorizes tags, assigns transactionId, remote control |

Connect URL shape: wss://cms.example.com/ocpp/{chargePointId}

Optional: subprotocol ocpp1.6, HTTP Basic Auth. After open → send BootNotification.

# Message shapes

Every frame is a JSON array:

| Type | Shape |
| --- | --- |
| CALL | [2, messageId, action, payload] |
| CALLRESULT | [3, messageId, payload] |
| CALLERROR | [4, messageId, errorCode, description, details] |

Either side can send a CALL; the other replies.

# Go online

1. WebSocket open

2. BootNotification → CMS returns status, currentTime, interval

3. If Accepted → StatusNotification for connector 0 and each 1…N

4. Heartbeat every interval seconds

| Boot field | Meaning |
| --- | --- |
| Vendor / Model / Serial / firmwareVersion | CP identity |
| conf status | Accepted / Pending / Rejected |
| conf interval | Heartbeat seconds |

Simulator defaults: Massive Mobility / Massive-CP-Sim-16 / Massive-CPS-16.3.2.1

# Connectors & status

| connectorId | Meaning |
| --- | --- |
| 0 | Whole charge point |
| 1…N | Outlets / guns |

Common statuses: Available → Preparing → Charging → Finishing → Available

Also: SuspendedEV, SuspendedEVSE, Reserved, Unavailable, Faulted

StatusNotification carries status, errorCode (NoError or fault), optional info.

# Auth & charging

idTag = RFID / app token string.

| Step | Message |
| --- | --- |
| Check tag | Authorize → idTagInfo.status (Accepted, Invalid, Blocked, Expired, ConcurrentTx) |
| Open session | StartTransaction → CMS returns transactionId |
| While charging | MeterValues (energy, power, SoC, …) |
| Close session | StopTransaction (reason: Local, Remote, EVDisconnected, EmergencyStop, …) |

Remote start: CMS RemoteStartTransaction → CP Authorize (if configured) + StartTransaction

Remote stop: CMS RemoteStopTransaction { transactionId }

Auth can be online CMS, local list, or cache. Same tag on two connectors often → ConcurrentTx / Blocked.

# CP → CMS (common)

BootNotification · Heartbeat · StatusNotification · Authorize · StartTransaction · MeterValues · StopTransaction · DataTransfer · FirmwareStatusNotification · DiagnosticsStatusNotification

# CMS → CP (common)

RemoteStart / RemoteStop · Reset · UnlockConnector · ChangeAvailability · Get/ChangeConfiguration · ClearCache · SendLocalList · ReserveNow / CancelReservation · TriggerMessage · GetDiagnostics · UpdateFirmware · Set/ClearChargingProfile · GetCompositeSchedule · DataTransfer

# Config keys worth knowing

| Key | Role |
| --- | --- |
| HeartbeatInterval | Heartbeat seconds |
| MeterValueSampleInterval | MeterValues seconds |
| MeterValuesSampledData | Which measurands |
| NumberOfConnectors | Outlet count |
| AuthorizeRemoteTxRequests | RemoteStart must Authorize? |
| SupportedFeatureProfiles | Advertised profiles |

ChangeConfiguration may return RebootRequired → needs Reset.

# Minimal working set

Must send: Boot, Heartbeat, Status, Authorize, Start, MeterValues, Stop

Must handle: RemoteStart/Stop, Reset, Get/ChangeConfiguration, TriggerMessage, ChangeAvailability

# Pitfalls

| Symptom | Likely cause |
| --- | --- |
| Online socket, CMS offline | Boot missing / Rejected / wrong cpId |
| Authorize Invalid | Tag not in CMS |
| Start Blocked / ConcurrentTx | Tag already charging elsewhere |
| No live kW | No MeterValues / wrong measurands |
| Stuck Finishing | Waiting for Available / unplug |
| Subprotocol error | CMS requires or forbids ocpp1.6 |

# Simulator note

Browser UI ≠ OCPP. Only the Node backend opens the WebSocket to the CMS. The UI drives the sim over REST / Socket.IO.